Computer Networks Interview Questions (200 Qs)

Basic Concepts & Components (Q1–Q40)

| Q# | Question | Answer / Example |
| --- | --- | --- |
| 1 | What is a computer network? | Collection of devices connected to share resources. Example: LAN in a school, Internet at home. |
| 2 | Name key network components and their roles | AP: Wireless connection point (Wi-Fi router). Gateway: Connects networks. Hub: Broadcasts data. Switch: Sends to specific MAC. Router: Routes between networks. Bridge: Connects segments. Repeater: Extends signal. |
| 3 | Difference between hub, switch, router, bridge, repeater, AP | Hub: Layer1 broadcast. Switch: Layer2 MAC unicast. Router: Layer3 IP routing. Bridge: Layer2 segment connection. Repeater: Signal booster. AP: Wireless connectivity. |
| 4 | Explain RTS/CTS in wireless networks | Request to Send / Clear to Send – avoids collision in Wi-Fi. |
| 5 | Star topology | Devices connect to central hub/switch. Example: Office LAN. |
| 6 | Bus topology | Single backbone cable. Example: Old Ethernet networks. |
| 7 | Ring topology | Devices connected in circular path. Example: Token Ring. |
| 8 | Mesh topology | Every device connects to every other. Example: Internet backbone. |
| 9 | Most fault-tolerant topology | Mesh; Star depends on central hub. |
| 10 | LAN vs MAN vs WAN vs PAN vs WLAN vs CAN | LAN: Wi-Fi in home, MAN: campus network, WAN: Internet, PAN: Bluetooth devices, WLAN: Wi-Fi network, CAN: automotive controller network. |
| 11 | What is an ad-hoc network? | Temporary wireless network without AP. Example: Wi-Fi Direct file sharing. |
| 12 | What is a hotspot? | AP providing Internet access. Example: Cafe Wi-Fi. |
| 13 | Types of wired networks | Ethernet, Fiber Optics. |
| 14 | Types of wireless networks | Wi-Fi, Bluetooth, Zigbee, 5G. |
| 15 | WLAN vs WWAN | WLAN: Local area Wi-Fi. WWAN: Wide area, e.g., mobile network. |
| 16 | OSI layers and functions | Physical, Data Link, Network, Transport, Session, Presentation, Application. |
| 17 | TCP/IP layers vs OSI layers | TCP/IP: Application, Transport, Internet, Network Access. |
| 18 | Protocol examples: Application layer | HTTP, FTP, SMTP. |
| 19 | Protocol examples: Transport layer | TCP (reliable), UDP (fast). |
| 20 | Protocol examples: Internet layer | IP, ICMP. |
| 21 | Protocol examples: Network Access layer | Ethernet, ARP. |
| 22 | TCP vs UDP | TCP: Connection-oriented, reliable (file transfer). UDP: Connectionless, faster (video streaming). |
| 23 | Explain sliding window | Multiple packets in transit; flow control. Example: TCP. |
| 24 | Stop-and-wait | Send one packet at a time, wait for ACK. |
| 25 | Go-Back-N | Retransmit all packets from lost packet. |
| 26 | Selective Repeat | Retransmit only lost packets. |
| 27 | IP address classes | A, B, C, D, E (A-C usable for hosts). |
| 28 | Classful vs Classless addressing | Classful: Fixed classes. Classless: CIDR allows flexible subnetting. |
| 29 | What is a subnet mask? | Identifies network portion. Example: 255.255.255.0 |
| 30 | Explain DHCP | Dynamically assigns IP addresses to devices. |
| 31 | Routing algorithms | Distance Vector (RIP), Link State (OSPF). |
| 32 | Handoff vs Handover | Switching between cells/towers in mobile networks. |
| 33 | Common port numbers | HTTP: 80, HTTPS: 443, FTP: 20/21, SMTP: 25, DNS: 53, DHCP: 67/68, POP3: 110, IMAP: 143. |
| 34 | What is ARP? | Maps IP to MAC. Example: IP 192.168.1.1 → MAC 00:0a:95:9d:68:16 |
| 35 | IP header fields | Version, IHL, TTL, Protocol, Source IP, Destination IP. |
| 36 | Types of wireless | Infrared, RF, Microwave, Satellite. |
| 37 | What is handover? | Switching between 4G towers while moving. |
| 38 | Ad-hoc vs Infrastructure vs Mesh Wi-Fi | Ad-hoc: no AP, Infrastructure: AP central, Mesh: devices relay traffic. |
| 39 | Multiplexing types | FDM, TDM, WDM. Example: Cable TV uses FDM. |
| 40 | Switching types | Circuit, Packet, Message. Internet uses packet switching. |

Intermediate Concepts (Q41–Q100)

Protocols & Error Control

| Q# | Question | Answer / Example |
| --- | --- | --- |
| 41 | Error detection types | Parity, CRC, Checksum. |
| 42 | ARQ protocols | Stop-and-Wait, Go-Back-N, Selective Repeat. |
| 43 | Stop-and-wait example | Send one packet, wait for ACK; TCP uses sliding window. |
| 44 | Go-Back-N example | Window size=4, packet 2 lost → retransmit 2,3,4. |
| 45 | Selective Repeat example | Only lost packet 2 retransmitted; others OK. |
| 46 | QoS definition | Ensures performance: low latency, guaranteed bandwidth, minimal packet loss. |
| 47 | QoS example | Zoom prioritizes video packets over downloads. |
| 48 | QoS concepts | Bandwidth allocation, traffic prioritization, latency/jitter control. |
| 49 | NAT | Translates private IP → public IP. Example: 192.168.x.x → Internet. |
| 50 | PAT | NAT + port mapping. Multiple devices share one public IP. |
| 51 | VPN | Secure encrypted tunnel over Internet. Example: Remote employee access. |
| 52 | VLAN | Logical network segmentation. Example: HR & Sales isolated on same switch. |
| 53 | SSL/TLS | Encrypts client-server communication. Example: HTTPS. |
| 54 | IPSec | Encrypts/authenticates IP packets. Example: VPNs. |
| 55 | Firewall | Filters traffic based on rules. Example: Block unauthorized access to server. |
| 56 | SDN | Separates control & data planes. Centralized, programmable. Example: Google DC routing. |
| 57 | IoT networking | Connects sensors/devices. Example: Smart home, industrial sensors. |
| 58 | IoT protocols | MQTT, CoAP, Zigbee, LoRaWAN. |
| 59 | Scenario: File transfer vs video streaming | File → TCP reliable, Video → UDP faster. |
| 60 | Scenario: Multiple users, limited bandwidth | QoS prioritizes critical apps. |
| 61 | Scenario: NAT in home router | Multiple devices access Internet via single public IP. |
| 62 | Scenario: VLAN in office | HR and Finance isolated using VLANs. |
| 63 | Scenario: SDN dynamic routing | Traffic redirected to least congested path in data center. |
| 64 | Scenario: IoT sensors in factory | MQTT sends data to cloud broker for monitoring. |
| 65 | Protocol port scenario | HTTP server uses 80, FTP server uses 21. |
| 66 | DHCP process | Discover → Offer → Request → ACK (DORA). |
| 67 | Handoff scenario | Moving vehicle switches between 5G towers seamlessly. |
| 68 | Difference: Handoff & Handover | Same concept: switching connection during mobility. |
| 69 | Scenario: Sliding window | Window size=4, packets sent: 1,2,3,4; ACK for 2 arrives → send 5. |
| 70 | Scenario: Stop-and-wait | Packet 1 sent, wait for ACK → send packet 2. |
| 71 | Scenario: Go-Back-N | Packet 3 lost → retransmit 3,4,5. |
| 72 | Scenario: Selective Repeat | Only lost packet 3 retransmitted. |
| 73 | Difference TCP vs UDP | TCP: Reliable, ordered. UDP: Fast, no ACK. |
| 74 | ARP operation example | IP 192.168.1.2 → Broadcast ARP → MAC resolved. |
| 75 | IP header Version | IPv4=4, IPv6=6. |
| 76 | IP header IHL | Length of header in 32-bit words. |
| 77 | IP header TTL | Max hops before discard. |
| 78 | IP header Protocol | Indicates transport layer (TCP/UDP). |
| 79 | IP header Source/Destination IP | Identifies sender & receiver. |
| 80 | Multiplexing scenario | Cable TV FDM: multiple channels on same cable. |
| 81 | Switching scenario | Internet packet switching delivers packets via IP. |
| 82 | Circuit switching example | Telephone call. |
| 83 | Message switching example | Store-and-forward emails. |
| 84 | Ad-hoc Wi-Fi example | Mobile devices share files directly. |
| 85 | Infrastructure Wi-Fi example | AP provides connectivity to multiple devices. |
| 86 | Mesh Wi-Fi example | Devices relay packets to improve coverage. |
| 87 | PAN example | Bluetooth keyboard & mouse connection. |
| 88 | MAN example | Campus network connects multiple buildings. |
| 89 | WAN example | Internet, ISP backbone. |
| 90 | WLAN example | Wi-Fi network in home or office. |
| 91 | CAN example | Automotive network connecting ECUs. |
| 92 | Star topology example | Office LAN: All PCs connected to central switch. |
| 93 | Bus topology example | Old Ethernet coax cable. |
| 94 | Ring topology example | Token Ring network. |
| 95 | Mesh topology example | Full mesh between routers in ISP network. |
| 96 | Fault tolerance example | Mesh tolerates multiple link failures. |
| 97 | Repeater example | Extends Wi-Fi range in home. |
| 98 | Bridge example | Connects two LAN segments at Layer 2. |
| 99 | Gateway example | Router connecting LAN to Internet. |
| 100 | Access point example | Wi-Fi router connecting laptops and phones. |

Advanced Concepts (Q101–Q150)

1. Routing & IP/Subnetting

| Q# | Question | Answer / Example |
| --- | --- | --- |
| 101 | Distance Vector vs Link State | DV: Each router shares distance info (RIP). LS: Routers know full topology (OSPF). |
| 102 | RIP example | Each router sends table every 30s. Max hops=15. |
| 103 | OSPF example | Routers compute shortest paths using Dijkstra’s algorithm. |
| 104 | Scenario: Calculate subnet | IP: 192.168.1.0/26 → Subnets: 64 hosts per subnet. |
| 105 | CIDR example | 10.0.0.0/22 → IP range 10.0.0.0–10.0.3.255. |
| 106 | Scenario: Hand-over | Moving mobile from tower A to B → seamless connectivity. |
| 107 | Scenario: Handoff vs Handover | Handoff: same tower cell, Handover: between cells/towers. |
| 108 | NAT example | 192.168.1.2 → Public IP 203.0.113.5. |
| 109 | PAT example | Multiple internal hosts share 1 public IP using different ports. |
| 110 | VLAN scenario | HR VLAN 10, Sales VLAN 20 → traffic isolated on same switch. |
| 111 | Scenario: Router forwarding | Packet with destination 192.168.2.5 → router looks up routing table → forwards to correct interface. |
| 112 | Scenario: Default gateway | Host sends IP 8.8.8.8 → forwarded via default gateway. |
| 113 | Subnet mask calculation | 255.255.255.192 → 64 hosts/subnet. |
| 114 | Supernetting example | Combine 192.168.0.0/24 + 192.168.1.0/24 → 192.168.0.0/23. |
| 115 | Broadcast vs Multicast vs Unicast | Broadcast: all, Multicast: group, Unicast: single. Example: Video stream via multicast. |
| 116 | Scenario: Routing table | Given IPs and interfaces, determine outgoing interface. |
| 117 | Static vs Dynamic routing | Static: admin configured. Dynamic: protocols like RIP, OSPF. |
| 118 | Scenario: RIP loop prevention | Split horizon + poison reverse prevents loops. |
| 119 | Scenario: OSPF area configuration | Divide routers into areas → reduces LSDB size. |
| 120 | Scenario: VRRP example | Redundant gateway fails → backup takes over automatically. |

2. TCP/UDP & Sliding Window Advanced

| Q# | Question | Answer / Example |
| --- | --- | --- |
| 121 | Sliding window detailed | Sender can send N packets before ACK. Example: Window=4, send 1–4, receive ACK2 → send 5. |
| 122 | Scenario: Stop-and-Wait ARQ | Packet lost → sender retransmits 1 packet only. |
| 123 | Go-Back-N ARQ scenario | Window=5, packet 3 lost → retransmit 3,4,5. |
| 124 | Selective Repeat ARQ scenario | Only lost packets retransmitted; others accepted. |
| 125 | TCP 3-way handshake | SYN → SYN-ACK → ACK. Example: Browser connects to web server. |
| 126 | TCP vs UDP advanced scenario | File transfer → TCP; Streaming video → UDP. |
| 127 | TCP congestion control example | Slow start: CWND doubles per RTT until threshold. |
| 128 | UDP scenario | DNS request uses UDP port 53. |
| 129 | TCP window scaling | Large transfer over high-latency network → increase window size. |
| 130 | Example: TCP retransmission timeout | Packet delayed beyond RTO → retransmit. |

3. Wireless & Mobile Networks

| Q# | Question | Answer / Example |
| --- | --- | --- |
| 131 | Handoff/handover calculation | Moving from tower A to B at 60 km/h → time to switch frequency. |
| 132 | Ad-hoc vs Infrastructure vs Mesh Wi-Fi | Ad-hoc: devices only. Infrastructure: AP-based. Mesh: devices relay for coverage. |
| 133 | Hotspot vs AP | Hotspot: provides Internet via AP. AP: wireless connection point. |
| 134 | Scenario: Wireless interference | Multiple APs overlapping → channel planning needed. |
| 135 | Scenario: Wi-Fi QoS | Prioritize VoIP packets over downloads using WMM. |
| 136 | Cellular network layers | MAC, PHY, RLC, PDCP, Core network. Example: LTE architecture. |
| 137 | IoT low-power protocol | Zigbee → 10–100m range, low energy. |
| 138 | IoT industrial example | MQTT broker collects sensor data for factory monitoring. |
| 139 | SDN scenario | Controller dynamically reroutes congested traffic in data center. |
| 140 | IoT scenario | Smart home devices send sensor info via MQTT → cloud → mobile app. |

4. Network Security

| Q# | Question | Answer / Example |
| --- | --- | --- |
| 141 | SSL/TLS handshake | Client hello → Server hello → Cert exchange → Key generation → Secure connection. |
| 142 | IPSec modes | Transport: encrypts payload; Tunnel: encrypts entire packet. |
| 143 | Firewall types | Packet filter, Stateful, Application layer firewall. |
| 144 | Scenario: HTTPS connection | Browser connects via port 443 → TLS handshake → encrypted session. |
| 145 | Scenario: VPN | Employee connects from home to company LAN via VPN. |
| 146 | Scenario: NAT & security | NAT hides internal IPs → adds security layer. |
| 147 | Scenario: VLAN security | HR VLAN 10 cannot access Finance VLAN 20. |
| 148 | Scenario: Intrusion detection | IDS detects unusual packet flood → alerts admin. |
| 149 | Scenario: DDOS mitigation | Rate-limiting, firewall rules, CDN absorption. |
| 150 | Scenario: Wireless security | WPA3 protects Wi-Fi from eavesdropping. |

Expert Concepts (Q151–Q200)

1. QoS & Performance

| Q# | Question | Answer / Example |
| --- | --- | --- |
| 151 | QoS metrics | Latency, jitter, packet loss, bandwidth. Example: Zoom video. |
| 152 | Scenario: QoS prioritization | VoIP packets > File downloads. |
| 153 | Bandwidth allocation example | 1 Mbps reserved for video conference → guaranteed quality. |
| 154 | Traffic shaping scenario | Limit email traffic during office hours to prioritize video calls. |
| 155 | Scenario: Differentiated Services | DSCP field marks packet priority. |

2. Multiplexing & Switching Expert

| Q# | Question | Answer / Example |
| --- | --- | --- |
| 156 | FDM example | Multiple radio channels on same medium. |
| 157 | TDM example | Digital telephony channels share line in time slots. |
| 158 | WDM example | Fiber-optic multiple wavelengths. |
| 159 | Circuit switching scenario | PSTN call → dedicated circuit. |
| 160 | Packet switching scenario | Internet packets sent independently. |
| 161 | Message switching scenario | Email store-and-forward. |
| 162 | Switching delay calculation | Transmission + propagation + queuing. |
| 163 | Example: Hybrid switching | MPLS combines packet & label switching. |
| 164 | Scenario: Channel allocation | Frequency reuse in cellular network. |
| 165 | Example: Multiplexing in IoT | Sensor data time-division multiplexed to cloud. |

3. Distributed Systems & IoT

| Q# | Question | Answer / Example |
| --- | --- | --- |
| 166 | Distributed system components | Nodes, middleware, clients, servers. |
| 167 | GFS example | Google File System distributes file chunks across nodes. |
| 168 | Blockchain nodes example | Each node validates transactions. |
| 169 | IoT scenario | Smart home: Alexa, sensors, lights communicate via MQTT. |
| 170 | Industrial IoT example | Factory sensors → MQTT broker → cloud → monitoring app. |
| 171 | SDN advanced scenario | Controller dynamically reroutes traffic based on congestion. |
| 172 | SDN advantage | Centralized, programmable, efficient traffic. |
| 173 | SDN disadvantage | Single point of failure if controller down. |
| 174 | IoT protocol comparison | MQTT (lightweight), CoAP (RESTful), Zigbee (low power). |
| 175 | IoT security scenario | Encrypt MQTT messages to protect sensor data. |

4. Network Calculations & Scenarios

| Q# | Question | Answer / Example |
| --- | --- | --- |
| 176 | Scenario: Sliding window throughput | Window=4, RTT=100ms, packet size=1KB → throughput=40KB/s. |
| 177 | Scenario: Go-Back-N retransmission count | Lost packet 3 → 3,4,5 retransmitted. |
| 178 | Scenario: Selective Repeat retransmission | Only lost packets retransmitted. |
| 179 | Scenario: Subnet calculation | IP 192.168.10.0/27 → 32 addresses → 30 hosts usable. |
| 180 | Scenario: Supernet calculation | Combine 192.168.0.0/24 + 192.168.1.0/24 → 192.168.0.0/23. |
| 181 | Scenario: VLAN isolation | HR VLAN cannot access Sales VLAN. |
| 182 | Scenario: NAT/PAT mapping | Internal 192.168.1.2:1024 → Public 203.0.113.5:5001 |
| 183 | Scenario: ARP resolution | IP 192.168.1.10 → broadcast → MAC resolved. |
| 184 | Scenario: DHCP lease | Host requests → DHCP server assigns 192.168.1.50 for 24h. |
| 185 | Scenario: Routing table lookup | Destination 10.0.2.5 → router forwards to interface 1. |
| 186 | Scenario: RIP hop calculation | Max hops=15 → unreachable if beyond. |
| 187 | Scenario: OSPF shortest path | Dijkstra → least cost path selected. |
| 188 | Scenario: QoS packet prioritization | VoIP packet sent before FTP download. |
| 189 | Scenario: Wi-Fi coverage planning | AP placement for full office coverage. |
| 190 | Scenario: Wireless interference mitigation | Channel selection, signal power adjustment. |
| 191 | Scenario: SDN dynamic load balancing | Congested link → controller reroutes flows. |
| 192 | Scenario: Firewall rule | Block port 23 (Telnet), allow port 80. |
| 193 | Scenario: SSL/TLS handshake calculation | Key exchange + certificate verification → secure session. |
| 194 | Scenario: IPSec tunnel | Encrypt payload for secure site-to-site VPN. |
| 195 | Scenario: IoT sensor data throughput | Sensor publishes 10 messages/sec → broker forwards to cloud. |
| 196 | Scenario: TCP congestion window calculation | CWND=4 packets, ACK received → CWND doubles in slow start. |
| 197 | Scenario: Packet loss recovery | Go-Back-N → retransmit all after lost packet. Selective Repeat → only lost packet. |
| 198 | Scenario: Mobile handover | Switching 4G → 5G while moving without dropping call. |
| 199 | Scenario: Network security breach | Intrusion detected → firewall blocks IP. |
| 200 | Scenario: End-to-end network example | Host A → Router1 → Router2 → Host B; calculate path, hops, latency. |